Міністерство освіти і науки України

Національний університет „Львівська політехніка”

Кафедра ЕОМ

Звіт

з лабораторної роботи №2

з дисципліни: “Моделювання комп’ютерних систем”

на тему: «Структурний опис цифрового автомата. Перевірка роботи автомата за допомогою стенда Elbert V2 – Spartan 3A FPGA.»

Виконав: ст.гр. КІ-201

Гірняк О.Р

Прийняв:

Козак Н. Б.

Львів

2023

Мета роботи: на базі стенда Elbert V2 – Spartan 3A FPGA реалізувати цифровий автомат світлових ефектів згідно вимог.

Завдання згідно з варіантом:

Виконання роботи:

1. Створив проект.

2. Додав VHDL Module для імплементації логіки формування вихідних сигналів.

Код OutputLogic:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx primitives in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity out_logic_intf is

Port ( IN_BUS : in  std_logic_vector(2 downto 0);

OUT_BUS : out  std_logic_vector(7 downto 0)

);

end out_logic_intf;

architecture out_logic_arch of out_logic_intf is

begin

OUT_BUS(0) <= (not(IN_BUS(2)) and not(IN_BUS(1)) and not(IN_BUS(0)));

OUT_BUS(1) <= ((not(IN_BUS(2)) and not(IN_BUS(1)) and not(IN_BUS(0))) or (not(IN_BUS(2)) and not(IN_BUS(1)) and (IN_BUS(0))));

OUT_BUS(2) <= ((not(IN_BUS(2)) and not(IN_BUS(1)) and (IN_BUS(0))) or (not(IN_BUS(2)) and (IN_BUS(1)) and not(IN_BUS(0))));

OUT_BUS(3) <= ((not(IN_BUS(2)) and (IN_BUS(1)) and not(IN_BUS(0))) or (not(IN_BUS(2)) and (IN_BUS(1)) and (IN_BUS(0))));

OUT_BUS(4) <= ((not(IN_BUS(2)) and (IN_BUS(1)) and (IN_BUS(0))) or ((IN_BUS(2)) and not(IN_BUS(1)) and not(IN_BUS(0))));

OUT_BUS(5) <= (((IN_BUS(2)) and not(IN_BUS(1)) and not(IN_BUS(0))) or ((IN_BUS(2)) and not(IN_BUS(1)) and (IN_BUS(0))));

OUT_BUS(6) <= (((IN_BUS(2)) and not(IN_BUS(1)) and (IN_BUS(0))) or ((IN_BUS(2)) and (IN_BUS(1)) and not(IN_BUS(0))));

OUT_BUS(7) <= (((IN_BUS(2)) and (IN_BUS(1)) and not(IN_BUS(0))) or ((IN_BUS(2)) and (IN_BUS(1)) and (IN_BUS(0))));

end out_logic_arch;

Рис. 1 Діаграма проведеної симуляції для OutputLogic

3. Додав VHDL Module для імплементації логіки формування переходів. Код TransitionLogic:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity transition_logic_intf is

Port ( CUR_STATE : in  std_logic_vector(2 downto 0);

MODE : in  std_logic;

RES : in  std_logic;

NEXT_STATE : out  std_logic_vector(2 downto 0)

);

end transition_logic_intf;

architecture transition_logic_arch of transition_logic_intf is

begin

NEXT_STATE(0) <= (not(RES) and not(MODE) and not(CUR_STATE(2)) and not(CUR_STATE(1)) and not(CUR_STATE(0))) or  -- 000 -> 001

(not(RES) and not(MODE) and not(CUR_STATE(2)) and CUR_STATE(1) and not(CUR_STATE(0))) or  -- 010 -> 011

(not(RES) and not(MODE) and CUR_STATE(2) and not(CUR_STATE(1)) and not(CUR_STATE(0))) or  -- 100 -> 101

(not(RES) and not(MODE) and CUR_STATE(2) and CUR_STATE(1) and not(CUR_STATE(0))) or   -- 110 -> 111

(not(RES) and MODE and not(CUR_STATE(2)) and (CUR_STATE(1)) and not(CUR_STATE(0))) or   -- 001 <- 010

(not(RES) and MODE and (CUR_STATE(2)) and not(CUR_STATE(1)) and not(CUR_STATE(0))) or   -- 011 <- 100

(not(RES) and MODE and CUR_STATE(2) and (CUR_STATE(1)) and not(CUR_STATE(0))) or   -- 101 <- 110

(not(RES) and MODE and not(CUR_STATE(2)) and not(CUR_STATE(1)) and not(CUR_STATE(0)));  -- 111 <- 000

NEXT_STATE(1) <=  (not(RES) and not(MODE) and not(CUR_STATE(2)) and not(CUR_STATE(1)) and CUR_STATE(0)) or  -- 001 -> 010

(not(RES) and not(MODE) and not(CUR_STATE(2)) and CUR_STATE(1) and not(CUR_STATE(0))) or  -- 010 -> 011

(not(RES) and not(MODE) and CUR_STATE(2) and not(CUR_STATE(1)) and CUR_STATE(0)) or   -- 101 -> 110

(not(RES) and not(MODE) and CUR_STATE(2) and CUR_STATE(1) and not(CUR_STATE(0))) or   -- 110 -> 111

(not(RES) and MODE and not(CUR_STATE(2)) and (CUR_STATE(1)) and (CUR_STATE(0))) or   -- 010 <- 011

(not(RES) and MODE and CUR_STATE(2) and not(CUR_STATE(1)) and not(CUR_STATE(0))) or   -- 011 <- 100

(not(RES) and MODE and CUR_STATE(2) and CUR_STATE(1) and CUR_STATE(0)) or    -- 110 <- 111

(not(RES) and MODE and not(CUR_STATE(2)) and not(CUR_STATE(1)) and not(CUR_STATE(0)));  -- 111 <- 000

NEXT_STATE(2) <=  (not(RES) and not(MODE) and not(CUR_STATE(2)) and CUR_STATE(1) and CUR_STATE(0)) or   -- 011 -> 100

(not(RES) and not(MODE) and CUR_STATE(2) and not(CUR_STATE(1)) and not(CUR_STATE(0))) or  -- 100 -> 101

(not(RES) and not(MODE) and CUR_STATE(2) and not(CUR_STATE(1)) and CUR_STATE(0)) or   -- 101 -> 110

(not(RES) and not(MODE) and CUR_STATE(2) and CUR_STATE(1) and not(CUR_STATE(0))) or   -- 110 -> 111

(not(RES) and MODE and CUR_STATE(2) and not(CUR_STATE(1)) and CUR_STATE(0)) or   -- 100 <- 101

(not(RES) and MODE and CUR_STATE(2) and CUR_STATE(1) and not(CUR_STATE(0))) or   -- 101 <- 110

(not(RES) and MODE and CUR_STATE(2) and CUR_STATE(1) and CUR_STATE(0)) or    -- 110 <- 111

(not(RES) and MODE and not(CUR_STATE(2)) and not(CUR_STATE(1)) and not(CUR_STATE(0)));   -- 111 <- 000

end transition_logic_arch;

Рис. 2 Діаграма проведеної симуляції для TransitionLogic

4. Згенерував Schematic файли для OutputLogic та TransitionLogic.

5. Створив Schematic файл LightController, реалізував в ньому пам’ять стану автомата та зв’язав між собою всі його частини.

Рис. 3 Схема LightController

Рис. 4 Діаграма проведеної симуляції для LightController

6. Створив Schematic файл TopLevel, в якому реалізував подільник вхідної частоти та інтегрував його зі створеним автоматом, попередньо згенерувавши для нього Schematic файл.

Рис. 5 Схема TopLevel

Рис. 6 Діаграма проведеної симуляції для TopLevel

7. Створив Constraints файл, зв’язав в ньому виводи схеми та фізичні виводи плати.

Вміст Constraints.ucf:

#+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++#

# This file is a .ucf for ElbertV2 Development Board                                                  #

# To use it in your project :                                                                         #

# * Remove or comment the lines corresponding to unused pins in the project                           #

# * Rename the used signals according to the your project                                             #

#+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++#

#**************************************************************************************************************************************************#

#                                            UCF for ElbertV2 Development Board                                                                    #

#**************************************************************************************************************************************************#

CONFIG VCCAUX = "3.3" ;

# Clock 12 MHz

NET "CLOCK"                  LOC = P129  | IOSTANDARD = LVCMOS33 | PERIOD = 12MHz;

####################################################################################################

#                                       LED

####################################################################################################

NET "OUT_BUS(0)"             LOC = P46   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(1)"             LOC = P47   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(2)"             LOC = P48   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(3)"             LOC = P49   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(4)"             LOC = P50   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(5)"             LOC = P51   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(6)"             LOC = P52   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "OUT_BUS(7)"             LOC = P53   | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

####################################################################################################

#                                   DP Switches

####################################################################################################

NET "MODE"        LOC = P70   | PULLUP  | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

####################################################################################################

#                                      Switches

####################################################################################################

NET "SREED"        LOC = P80   | PULLUP  | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

NET "RESET"        LOC = P79   | PULLUP  | IOSTANDARD = LVCMOS33 | SLEW = SLOW | DRIVE = 12;

8. Згенерував бінарний файл для цифрового автомата світлових ефектів.

Висновок: виконавши лабораторну роботу, здобуто навики реалізації цифрових автоматів світлових ефектів.